#!/bin/bash

# Interface Definitions

INTERNET="ens33"   # Connection to NAT/Internet

SERVER="ens38"     # Connection to DNS Server (VMNET3 - 192.168.1.254)

ROUTER1="ens37"    # Connection to Router 2 (VMNET2 - 192.168.2.1)

CLIENT="ens39"     # Connection to Client (VMNET4 - 192.168.3.254)

# Flush old rules and delete user-defined chains

sudo iptables -F

sudo iptables -t nat -F

sudo iptables -X

########################### STATIC ROUTING ###########################

# Add routes to networks behind Router 2

sudo ip route add 192.168.5.0/24 via 192.168.2.1

sudo ip route add 192.168.4.0/24 via 192.168.2.1

########################### NAT ###########################

# Enable masquerading for outgoing internet traffic

sudo iptables -t nat -A POSTROUTING -o $INTERNET -j MASQUERADE

########################### INPUT ###########################

# Allow SSH access from a specific management IP

sudo iptables -A INPUT -p tcp -s 192.168.21.1 -i $INTERNET --dport 22 -j ACCEPT

########################### RELATED ESTABLISHED ###########################

# Allow return traffic for existing connections

sudo iptables -A OUTPUT -m state --state RELATED,ESTABLISHED -j ACCEPT

sudo iptables -A INPUT -m state --state RELATED,ESTABLISHED -j ACCEPT

sudo iptables -A FORWARD -m state --state RELATED,ESTABLISHED -j ACCEPT

########################### FORWARD ###########################

# Condition 2: Client 3.1 can access DNS Server 1.1 (UDP 53)

sudo iptables -A FORWARD -p udp --dport 53 -s 192.168.3.1 -d 192.168.1.1 -i $CLIENT -o $SERVER -j ACCEPT

# Condition 2: Client 4.1 (via Router 2) can access DNS Server 1.1

sudo iptables -A FORWARD -p udp --dport 53 -s 192.168.4.1 -d 192.168.1.1 -i $ROUTER1 -o $SERVER -j ACCEPT

# Condition 4: DNS Server 1.1 can access Internet (UDP 53)

sudo iptables -A FORWARD -p udp --dport 53 -s 192.168.1.1 -d 8.8.8.8 -i $SERVER -o $INTERNET -j ACCEPT

# Condition 3: Client 3.1 can access Internet (Port 80, 443)

sudo iptables -A FORWARD -p tcp --dport 80 -s 192.168.3.1 -i $CLIENT -o $INTERNET -j ACCEPT

sudo iptables -A FORWARD -p tcp --dport 443 -s 192.168.3.1 -i $CLIENT -o $INTERNET -j ACCEPT

# Condition 1: Client 3.1 can access Web Server 5.1 (Port 80)

sudo iptables -A FORWARD -p tcp --dport 80 -s 192.168.3.1 -d 192.168.5.1 -i $CLIENT -o $ROUTER1 -j ACCEPT

########################### POLICY ###########################

# Block all other traffic by default (Whitelist approach)

sudo iptables -P INPUT DROP

sudo iptables -P OUTPUT DROP

sudo iptables -P FORWARD DROP